# INFER user guide

Slides: [INFER-user-guide.pptx](INFER-user-guide.pptx) (figures under [slides/assets](slides/assets)).

Intelligent Navigation and Facility Environment Reasoning

This guide is for people using the INFER workspace: opening a building, reading the 3D / plan / graph, asking for a walkable route, running a what-if, and sharing that route. It describes the current internship proof-of-concept, not a finished product.

INFER turns a static building file (IFC or IndoorGML) into something you can route through: rooms, doors, stairs, lifts, furniture obstacles, and exits. It does not change the original BIM file. Anything the software *guesses* (for example an extra door–room link) stays labelled as inferred.

Everyday routing runs on a per-storey walkability grid (navmesh), not by clicking nodes on the Graph Viewer. The graph is for understanding connectivity and for removing entities from the live model.

# 1. What you need

A PC with Python 3.11+ and Node 20+.

The backend on http://127.0.0.1:8000.

The frontend in a browser (npm run dev — typically http://localhost:5173 or another Vite port).

An `.ifc`, `.ifczip`, `.gml`, or `.indoorgml` file.

Models stay on this machine under data/. Do not upload facility IFCs to public cloud BIM tools or public AI services.

## Start the two processes

Backend (PowerShell):

cd backend
.\.venv\Scripts\Activate.ps1
uvicorn app.main:app --host 127.0.0.1 --port 8000 --reload

Frontend:

cd frontend
npm run dev

If the page loads but ingest fails, the API is usually not running. Check http://127.0.0.1:8000/health.

# 2. The workspace

Three panes sit side by side. Open or close them from the pane bar; drag the splits to resize.

| Pane | What it is for |
| --- | --- |
| **3D Viewer** | Orbit or fly the BIM. Optional navmesh overlay. Storey clip is independent of the 2D level. |
| **2D Viewer** | One storey at a time: architectural plan (**IFC** tab) or walkable regions (**Navmesh** tab). |
| **Graph Viewer** | Rooms (and stairs/lifts) as a network. Most door nodes are hidden so the picture is readable. |

Reload the page resets pane sizes to equal thirds and clears the loaded model from memory. Derived files remain on disk; you still ingest (or re-select) to populate the UI.

Light / dark theme is the sun/moon control on the top bar.

Control is the collapsible tray on the right: search rooms, doors, heals, exits, and stairs by level and name; remove or restore without deleting the file.

# 3. Open a model

Top bar → model name (says No model loaded until you have one) → Open model…. Empty panes also have an Open model button.

Drop a file or browse. Supported: .ifc, .ifczip, .gml, .indoorgml.

INFER then, in order:

Loads IFC for 3D (IndoorGML has no BIM mesh — 3D falls back to extruded footprints).

Uploads the file to the backend.

Extracts spaces, doors, stairs, lifts, exit candidates.

Builds 2D footprints (room outlines, walls, doors, furniture occupancy).

Builds a connectivity graph. After ingest the live graph is Geometry rules (IFC relations plus healing so sparse exports still connect).

A large building can take one to several minutes, especially footprints (furniture meshes). Watch the dialog and the top-bar status line.

When it finishes you should see spaces on the floorplan and a toast with space / link counts.

# 4. Floorplan — IFC tab (the drawing)

Pick the storey from the dropdown. The 2D plan is one floor at a time.

IFC is the architectural view: rooms, walls, door glyphs (swing/slide when the file provides it), stairs, cream furniture, and a route overlay if you already have a navmesh path.

## Camera

Left-drag — pan.

Scroll — zoom.

Shift + scroll — rotate the plan.

Fit — frame the current storey.

A camera disc on the plan tracks the 3D viewpoint when the 3D pane is open (heading cone while you fly).

## Legend

Toggle layers: spaces, walls, doors, stairs, furniture, route. Turn furniture off if the plan is busy; routing still treats those shapes as obstacles when they are in the footprints.

Left-click a room on the IFC tab to select it (same idea as Graph / Control).

# 5. How to read the drawings

| You see | Meaning |
| --- | --- |
| Grey room fill | Walkable space from the IFC space (or heal). Holes are voids (atria). |
| Thick wall poché | Wall footprint. Doorways are punched visually. |
| Cream furniture | True occupancy of desks, screens, chairs — **not** a rubber-band box around the cubicle. Chair wells stay empty and walkable. |
| Amber door glyph | IFC door. |
| Dashed stair | Stair overlay (plan hull). |
| Coloured route | Walkable path after smoothing (not the raw grid staircase). |
| Blue camera disc | Where the 3D camera is looking on this storey. |

Tiny fixtures (clocks, frames) are dropped so they do not clutter routing.

# 6. 3D Viewer

Orbit — drag to rotate around the building.

Fly — WASD to move, Space up, Shift down, hold Ctrl to go faster.

IFC / Navmesh — real BIM mesh, or a simplified portal/navmesh view of the walkable model.

Storey filter — clip which floors you see in 3D. Independent of the 2D storey dropdown.

Clicking a Worst bottlenecks row (when evacuation load is on) flies 3D to that door or stair as well as switching the 2D storey.

# 7. Graph Viewer

Rooms (and stairs/lifts) as nodes, laid out by storey. Everyday paths are not picked here — use Navmesh right-click on the plan.

Scroll — zoom. Drag background — pan. Fit — whole graph.

Left-click a space or link — select (multi-select is allowed).

Right-click a node — remove or restore that entity from the live graph (soft exclude). The room can still show as a ghost on the plan.

Right-click a link — disable or restore (disabled links look dashed).

Graph variant (dropdown) — only these two appear in the UI:

| Variant | What it is |
| --- | --- |
| **Geometry rules** (default after ingest) | IFC plus healed door / opening / stair links. Use this while routing. |
| **IFC relations** | Only explicit IFC space-boundary relations. Often too sparse to connect a real export. |

Healed links are coloured: yellow door heal, green space opening, purple stair. Grey is an authored IFC relation. Nested “parent” rooms can show a red circle (a large space that contains smaller ones).

There is no Topologic / TopologicPy choice in the operator UI. That backend variant is an unfinished optional experiment, not part of a demo.

# 8. Control panel

The Control tray (right):

Filter by level and category (region, IFC door, door heal, space heal, exit, stair/lift).

Search names.

See the current selection.

Remove or restore (same idea as Graph Viewer right-click).

Use Control when you need a door by name instead of hunting on the plan.

Remove vs block: Remove takes the entity out of the live graph/navmesh until you restore it. Block (next section) is a session what-if: the entity stays drawn, with a slash.

# 9. Ask for a walkable route (Navmesh tab)

Switch the 2D Viewer to Navmesh. You should see walkable regions, door/heal/exit portals, and (if enabled) furniture.

If the bar says Recalculating navmesh…, wait — exclusions and first load rebuild the grids.

## Point to point (**Route**)

Choose Route.

Right-click a walkable spot — start pin.

Right-click another spot — end pin (can be on another storey; stairs/lifts bridge floors).

The line stays in rooms, goes through doors, around furniture, and tries to stay about 40 cm off walls so it does not scrape inner corners. In a very narrow pinch it may get closer because there is no wider option.

Long right-click — clear the route.

Click inside a tinted region. A miss does nothing useful.

## Nearest exit (**Nearest exit**)

Choose Nearest exit.

Right-click one point.

INFER searches for the closest reachable exit, including via stairs to another floor.

## Selection vs block

On Navmesh:

Left-click a portal or room — select after a short delay (Inspector / Control).

Double-click a portal — block / unblock that door or opening for this session (slash on the circle).

Double-click a space — block / unblock the whole room (pale fill + diagonal slash). Routing and evacuation skip it; the room is not removed.

The status chip N blocked · clear clears all blocked doors and rooms at once. The IFC file is unchanged.

# 10. Evacuation load

Still on Navmesh, turn on Evacuation load.

INFER pretends every unblocked room walks to its nearest exit and heats the doors and stairs that would carry the most traffic. Larger rooms count for more than tiny ones (~area, not a real headcount file).

You may see computing… on a big building.

Blocking a door or a room changes the heat.

Worst bottlenecks (top-right list): ranked doors/stairs; click a row to jump 2D storey and fly 3D to that opening.

This is a planning sketch, not a certified fire-engineering model.

# 11. Share and export a route

When a navmesh route exists (start and end, or nearest-exit path), the top bar shows Share.

GLB download (route tube plus rooms, or live IFC geometry if the 3D pane is open and loaded).

On iPhone, USDZ for Quick Look / AR when offered.

Short-lived share link + QR so another device on the same network can open the 3D path without installing INFER.

If the 3D pane is closed, export uses a simpler extruded-plan stand-in, not full BIM materials.

# 12. Limits (read this before a demo)

POC quality. Paths, heals, and exits are for demonstration. Do not use them as the sole basis for life-safety sign-off.

IFC quality matters. Missing spaces, missing doors, or furniture with no usable mesh leave gaps.

Refresh loses the UI session. Re-open the model.

One storey on the 2D plan. Change the dropdown to see another floor; a multi-storey *route* still draws the piece on the storey you are viewing.

Walls on the IFC plan can still look like simple strips (hulls). Furniture uses true concave occupancy.

A few odd furniture types may have no extractable plan outline and will not draw or block.

# 13. Quick recipes

“Show me the building.”

Open model → wait for toast → 2D IFC tab, pick a storey → Legend if cluttered → 3D Orbit or Fly.

“Get me from this desk to the lobby.”

Navmesh → Route → right-click desk well → switch storey if needed → right-click lobby.

“Nearest way out from here.”

Navmesh → Nearest exit → right-click.

“This door is locked” / “This room is closed.”

Navmesh → double-click the portal or the space → re-run the route or refresh evacuation load. Unblock the same way, or N blocked · clear.

“This stair should not exist in the live model.”

Control or Graph → Remove the stair (not the same as a session block). Restore from Control / Graph.

“Where would people bunch up?”

Navmesh → Evacuation load → read the heat and Worst bottlenecks.

“Why won’t it route?”

You are on the IFC tab (no pins). Or the click missed a region. Or start and end are disconnected (blocked door/room, removed entity, sparse IFC relations graph). Switch to Geometry rules, unblock, pick points inside rooms.

# 14. For developers (short)

| Need | Where |
| --- | --- |
| Run / API | [README.md](../README.md), [backend/README.md](../backend/README.md) |
| Graph variants (including unused topologic API) | [graph-variants.md](graph-variants.md) |
| Graph pane internals | [graph-viewer.md](graph-viewer.md) |
| Interactive API | http://127.0.0.1:8000/docs |

INFER is an internal government internship proof-of-concept. Not licensed for external distribution.